#### Section 1: Fundamentals of Wireless Security

- What is the main purpose of Wireless Device Security?

The main purpose of wireless device security is to protect wireless networks from unauthorized access, misuse, and disruption

- Explain the difference between WEP (Wired Equivalent Privacy) and WPA2.

WEP is an outdated and insecure Wi-Fi encryption protocol, while WPA2 provides strong, modern encryption using AES, making it far more secure for wireless networks

- What role does AES encryption play in wireless security?

AES encryption provides a robust protection for data transmitted over wireless networks

- What are rogue access points, and why are they considered a security threat?

A rouge Access Point is a wireless access point that connects to a network without explicit authorisation normally installed by insiders or malicious attackers.

- How can wireless network vulnerability scanning help mitigate potential security risks?

#### Identifying and detecting vulnerabilities: Tools like OpenVAS can identify weaknesses, misconfigurations, and potential attack vectors within a network.

#### Preventing unauthorised access: By detecting rogue access points and weak encryption protocols, organizations can minimize the risk of unauthorized access.

#### Enhancing overall security posture: Regular scanning helps organizations strengthen their security measures, reducing the likelihood of data breaches.

#### 1Section 2: Wi-Fi Standards and Protocols

- Describe the significance of the 802.11 standards in Wi-Fi.

they define how wireless devices communicate and connect. Developed by the IEEE, these standards specify the MAC and physical layer protocols for wireless local area networks (WLANs)

- What is the difference between 802.11ac (Wi-Fi 5) and 802.11ax (Wi-Fi 6) in terms of speed and frequency?

Wi-Fi 6 offers faster speeds and improved throughput  capabilities, operates at both 5GHz and 2.4 GH and introduces features like OFDMA and MU-MIMO allowing multiple devices to transmit simultaneously enhancing performance in crowded environments.

Wifi 5 mainly uses 5GHz band

- Explain the concept of Service Set Identifier (SSID).

It is a unique name assigned to a wireless local area network (WLAN), allowing devices to identify and connect to the intended network amidst multiple overlapping networks normally consisting of 32 alphanumeric characters

- What is CSMA/CA (Carrier Sense Multiple Access/Collision Avoidance), and how does it work in Wi-Fi networks?

It’s a medium control protocol used in WI-FI networks to prevent data collisions, it works by allowing devices to listen to the channel before transmitting ensuring that only one device can send data at a time.

#### Section 3: Wireless LAN Components

- What is the function of an Access Point (AP) in a wireless LAN?

It connects to wireless devices allowing WIFI enabled devices to connect to wireless networks., extends the coverage of the network, allows centralised management, better network performance and gives advanced security features

- How does a Wi-Fi controller manage multiple access points in a network?

It acts as a centralised platform for configuration, security and monitoring. It also helps with load balancing to prevent congestion and optimal performance. Automatic channel management can be done to also improve signal quality.

- Differentiate between Ad-Hoc and Infrastructure modes in Wi-Fi networks.

AD-HOC mode allows devices to connect directly to each other without a central access point while infrastructure mode uses a central access point to manage communication and provide network connectivity.

- What are the characteristics of mesh networks, and where are they typically used?

Mesh networks have a decentralised structure where every device is directly connected to every other device allowing multiple paths of data transmission

#### Section 4: Wireless Frequencies and Modulation

- What is the difference between the 2.4 GHz and 5 GHz frequency bands in Wi-Fi networks?

2.4 GHz gives longer ranges but lower speeds while 5GHz provides faster speeds with shorter coverage and less interference

- How does signal attenuation affect wireless communication?

It affects wireless communication by causing a reduction in signal reducing speeds and slower downloads especially while distance increases.

- Explain the concept of modulation in wireless communication and its importance.

Modulation is a process of converting data into signals that can be transmitted over a medium. It involves altering the characteristics of a carrier signal to transmit more efficiently.

#### Section 5: Wi-Fi Security Threats and Mitigation

- What are the risks associated with using public open Wi-Fi networks?

Eavesdropping, man-in-the-middle attacks, identity theft, rouge wi-fi networks and malware infections

- How can bandwidth management help prevent denial of service attacks on a wireless network?

Controlling traffic flows of data, traffic filtering, rate limiting, mitigating DDos Effects and priority critical traffic processing

- Why is it important to disable unneeded wireless networks and filter wireless traffic?

Enhanced security, conservation of battery life, reduced expose to risks, minimised health risks and improved device performance.

#### Section 6: Advanced Wireless Security Measures

- How do VPNs enhance security in wireless networks, particularly for personal or untrusted devices?

Encryption, tunnelling protocols, user authentication, server authentication and access control