# FHSS vs DSSS

Connor Campbell

West Lothian College

Cyber HND 1

D.Reid

Author Note

Contents

FHSS vs DSSS	1

History of using spread spectrum in wireless transmissions	2

Design of Frequency-hopping spread spectrum	2

Uses of Frequency-hopping Spread Spectrum	2

Advantages of Frequency-hopping Spread Spectrum	2

Disadvantages of Frequency-hopping Spread Spectrum	2

Design of Direct-sequence Spread spectrum	2

Uses of Direct-sequence Spread Spectrum	2

Advantages of Direct-sequence Spread Spectrum	2

Disadvantages of Direct-sequence Spread Spectrum	2

Comparison of Frequency-hopping vs Direct-sequence	2

Conclusion	2

## Introduction

Spread spectrum is a wireless transmission technique where a signal is deliberately spread across a wider range of frequencies than would normally be required to transmit the data.

The main reasons for doing this include:

- reducing the effects of interference;

- making communications more resistant to jamming;

- allowing multiple devices to share radio spectrum;

- improving reliability in environments where signals can be disrupted.

Two important spread-spectrum techniques are Frequency-Hopping Spread Spectrum (FHSS) and Direct-Sequence Spread Spectrum (DSSS). They achieve similar goals but spread the transmitted signal in very different ways.

## History of using spread spectrum in wireless transmissions

Spread-spectrum technology has strong links to military communications.

During the early 20th century, engineers investigated methods of making radio transmissions harder to intercept or disrupt. A famous development came during World War II, when actress and inventor Hedy Lamarr and composer George Antheil patented a frequency-hopping communication system.

Their concept involved the transmitter and receiver changing frequencies together. This could make radio-controlled torpedoes more difficult for an enemy to jam.

Spread-spectrum concepts were subsequently developed extensively for military and secure communication systems.

They eventually became important in civilian wireless technology as well. Spread-spectrum techniques have been used in technologies including:

- early Wi-Fi standards;

- Bluetooth;

- GPS;

- CDMA cellular networks;

- cordless phones;

- wireless sensor and industrial communication systems.

## Design of Frequency-hopping spread spectrum

FHSS works by repeatedly changing the carrier frequency used to transmit a signal.

Instead of remaining on one frequency, the transmitter "hops" between a collection of frequency channels.

The transmitter and receiver both know the hopping sequence and timing. Therefore, when the transmitter changes to another frequency, the receiver changes with it.

The period spent on each frequency is called the dwell time.

If interference affects one frequency, only the part of the transmission occurring while that frequency is being used may be affected.

## Uses of Frequency-hopping Spread Spectrum

FHSS has been used in several wireless communication technologies.

A particularly important example is Bluetooth Classic, which operates within the 2.4 GHz ISM band and uses frequency hopping.

FHSS has also been used for:

- military communications;

- industrial wireless systems;

- older IEEE 802.11 wireless networks;

- some cordless telephones;

- remote-control systems;

- environments containing significant radio interference.

Bluetooth also uses Adaptive Frequency Hopping (AFH), which can reduce the use of channels experiencing interference.

## Advantages of Frequency-hopping Spread Spectrum

One of the biggest advantages is resistance to narrowband interference. If interference exists on one frequency, the FHSS system will soon move onto another frequency.

It can therefore provide good reliability in noisy radio environments.

FHSS can also offer:

Resistance to jamming: An attacker attempting to jam one frequency will only interfere with the communication temporarily unless they can interfere with a much larger part of the spectrum.

Reduced persistent interference: Multiple devices can potentially operate in the same frequency range because they do not continuously occupy the same channel.

Interception difficulty: A receiver that doesn't know the hopping pattern may find following the complete transmission more difficult.

However, be careful here: FHSS is not encryption. Frequency hopping alone should not be treated as providing confidentiality.

## Disadvantages of Frequency-hopping Spread Spectrum

The transmitter and receiver need to remain synchronised. If they lose track of the hopping sequence, communication can fail.

Other disadvantages include:

Complexity: Frequency-hopping hardware and protocols need to rapidly change frequencies while maintaining communication.

Bandwidth usage: The system requires access to multiple frequency channels.

Potentially lower data rates: Depending on the implementation, FHSS can offer lower throughput than other approaches.

Interference still occurs: Hopping reduces the impact of interference but does not eliminate it. The signal can still encounter occupied or noisy frequencies.

## Design of Direct-sequence Spread spectrum

DSSS works very differently.

Rather than rapidly changing carrier frequencies, DSSS combines the original data with a much faster pseudo-random sequence, often called a chipping code or spreading code.

Each original data bit is represented by several smaller units called chips.

Instead of transmitting one simple bit, the information is spread using the longer sequence.

At the receiver, the same spreading code is used to de-spread the received signal and recover the original information.

Because the chip rate is greater than the original data rate, the resulting transmission occupies a wider bandwidth.

## Uses of Direct-sequence Spread Spectrum

DSSS has been important in several major communication systems.

One of the best examples is the original IEEE 802.11 Wi-Fi standard, which supported DSSS in the 2.4 GHz band. 802.11b subsequently used a related coding technique called Complementary Code Keying (CCK).

Spread-spectrum/code-based techniques are also important in:

- GPS;

- CDMA cellular communications;

- satellite communications;

- military communication and navigation systems.

## Advantages of Direct-sequence Spread Spectrum

DSSS can provide strong resistance to certain forms of interference.

At the receiver, the correct spreading sequence helps distinguish the wanted signal from unrelated signals and noise.

Advantages include:

Interference resistance: Narrowband interference affects only part of the wider spread signal.

Noise resistance: De-spreading can improve the receiver's ability to recover the desired information from background noise.

Multiple access: Different spreading codes can allow multiple users to share spectrum. This principle became particularly important in CDMA systems.

Low power spectral density: The transmitted energy is distributed across a wider bandwidth rather than concentrated into a narrow channel.

## Disadvantages of Direct-sequence Spread Spectrum

DSSS requires more bandwidth than would be needed to transmit the original narrowband data.

It also requires additional processing.

The transmitter has to spread the information using the code, while the receiver must correctly synchronise with the code and despread the signal.

Other disadvantages include:

- increased receiver/transmitter complexity;

- accurate code synchronisation requirements;

- inefficient spectrum use in some situations;

- performance can deteriorate when interference becomes sufficiently strong;

- spreading codes alone do not provide proper encryption or security.

## Comparison of Frequency-hopping vs Direct-sequence

| Feature | FHSS | DSSS |
| --- | --- | --- |
| Basic technique | Changes carrier frequency repeatedly | Spreads data using a high-rate code |
| Frequency behaviour | Hops between channels | Uses a wider channel continuously |
| Interference resistance | Avoids remaining on an affected frequency | Spreads interference's effect across the encoded signal |
| Synchronisation | Hopping sequence/time | Spreading code/time |
| Complexity | Requires frequency hopping | Requires spreading/de-spreading |
| Historical Wi-Fi use | Early 802.11 option | Early 802.11 and 802.11b |
| Bluetooth | Yes, frequency hopping | Not traditional DSSS |
| Security | Hopping can make casual interception harder, but isn't encryption | Code can make casual detection harder, but isn't encryption |
| Main strength | Resilience by moving between frequencies | Resilience through spreading and processing gain |

## Conclusion

both FHSS and DSSS were developed to make wireless communication more robust, but they approach the problem differently.

FHSS repeatedly changes frequencies, meaning interference on an individual channel only affects the system temporarily. DSSS instead combines information with a high-rate spreading sequence and transmits the resulting signal across a wider bandwidth.

Neither approach is better than eachother, both have their own specific uses based on the surrounding factors like the available spectrum, required data rate, interference environment, hardware complexity and intended application.