- What is the main function of a Router?

take incoming packets, look at the intended destination network, then forward these packets to another network that is considered to be the best route towards the destination network

- What is the name of the file in RAM that stores the information used for the routing process?

Routing Table

- How are the contents of the routing table generally used by the router?

It uses the contents to identify what the information end destination is and where it should end with the best route by looking up the table and seeing what has the lowest hop count

- Name the three general ways a routing table is populated.

Directly Connected Routes

These are the networks that are directly connected to the router ports

Static Routes

These are manually configured by the network administrator

Dynamic Routes

These are learned through an appropriate routing protocol such as RIP, EIGRP or OSPF etc

- How are Static Routes populated in the routing table?

Manual configuration which specifies the end location network, subnet mask and the next ip to hop to.

- How are Dynamic Routes learned?

Its learned using continues communication between routers with dynamic routing protocols where it discovers nearby networks and looks for the best path using a specified metric

- In the structure of a routing table entry, what does the Administrative Distance identify?

It identifies how reliable it is, the lower the distance is more preferred route.

- What does the outgoing interface identify in a routing table entry?

It identifies the specific network connection (Ethernet as an example), that a router should use to forward the packet to its next destination. It also tells the router which port to send data out of for it to reach the next hop

- What is one thing that remains the same for a packet throughout its entire journey across routers?

The Final destination / IP address

- What year was IPv4 introduced with RFC 791?

1981

- What was the approximate number of addresses available in the IPv4 address space?

4 billion

- Name three inventive schemes used to conserve the IPv4 address space.

Private Addresses

Classless Addressing (CIDR – Classless Inter-Domain Routing)

VLSM (Variable Length Subnet Masks)

- In what year was IPv6 introduced?

1995

- Why are migration techniques needed for IPv6?

It is not backwards compatible with IPv4

- Name the three main categories of IPv4 and IPv6 migration techniques.

Dual Stack

Tunnelling

Translation

- Which migration technique allows devices to run both IPv4 and IPv6 protocol stacks simultaneously on the same network?

Dual Stack

- Describe how Tunnelling works to transport an IPv6 packet over an IPv4 network.

It transports an IPv6 packets over a IPv4 network by putting the IPv6 packed encapsulated inside an IPv4 packet

- How many bits does an IPv6 address use, and what is the total size of the address space expressed as a power of 2?

128 (2128)

- How are the 128-bit IPv6 addresses written down (format)?

The 128 bit addresses are written down using 8 groups of 4 digit hexadecimal numbers separated by a ‘:’

3FFE:085B:1F1F:0000:0000:0000:00A9:1234

- Name the three types of IPv6 addresses.

Unicast

Multicast

anycast