- What happens to the power level at the -3dB point if the input power to a cable is 80 Watts?

The −3 dB point means the power has dropped to half of its original value (because a −3 dB change corresponds to a factor of 0.5 in power).

- Which type of fibre optic cable is suitable for the longest distance run, and why?

single-mode fiber designed for long-distance transmission, allowing a single light mode to travel through its core, minimizing signal loss over extensive distances.

- What methods can be used to reduce noise interference in UTP media?

Cable Management, Proper Grounding, Shielded cables, avoid parallel running, surge protectors

- What is crosstalk in networking, and how can it be prevented?

Crosstalk in networking refers to unwanted interference that occurs when the electric or magnetic fields from one signal channel disrupt adjacent channels.

Cable Management, Proper Grounding, Shielded cables, avoid parallel running, surge protectors

- What are the differences between the EIA/TIA 568A and 568B wiring standards?

The primary difference between EIA/TIA 568A and 568B wiring standards lies in the arrangement of the wire pairs, specifically the positions of the green and orange pairs.

- What type of media does 100BaseTX use, and what are its characteristics?

100BaseTX is an Ethernet standard that uses twisted pair cables (typically Category 5 or higher) to transmit data at speeds of up to 100 Mbps.

- What does MDI/MDIX mean in relation to switch ports, and why is it useful?

MDI (Medium Dependent Interface) and MDIX (Medium Dependent Interface Crossover) are Ethernet port types that define how transmit (TX) and receive (RX) pairs are wired.

- What does DCE stand for in router technology, and what role does it play?

DCE stands for Data Circuit Terminating Equipment in router technology. It plays a crucial role in facilitating the transmission of data across communication networks.

- What is the maximum recommended distance for 1000BaseTX Ethernet cables?

100 meters

- What features does the Point-to-Point Protocol (PPP) provide in WAN connections?

Encapsulation, error detection, authentication, loop detection, multilink support and compression

- Why is it important to leave as short a length of cable untwisted as possible at the termination block?

to maintain signal integrity and reduce crosstalk

- What command-line utility can be used to confirm network communication between devices in the same subnet?

Ping

- What tool is commonly used to test and diagnose faults in fibre optic cables?

Optical Time-Domain Reflectometer (OTDR)

- What type of cable should be used to connect two computers directly without networking devices?

crossover cable.

- If a device has an IP address of 192.168.8.9/30, what must the other device’s IP address be to be in the same subnet?

192.168.8.10/30

- What are the main features of a client-server network, and how do they differ from peer-to-peer networks?

Centralized Management: A single server manages resources and services, allowing multiple clients to access them.

Resource Sharing: Clients can request and access shared resources, such as files and databases, from the server.

One-to-Many Connection: A single server can serve many clients simultaneously, making it efficient for large-scale systems.

Single Point of Failure: If the server fails, it can lead to a loss of service for all clients connected to it.

peer-to-peer networks do not have a central server. Instead, each device can act as both a client and a server, allowing for more decentralized communication and resource sharing. This means that

- What device is required to enable communication between two different subnets?

Router

- How can you determine if a straight-through cable is incorrectly used in a network?

Ensure that both ends of the cable follow the same wiring standard (T568A or T568B). If the wiring order is identical, the cable is straight-through.

- What is the function of DHCP in a network, and how does it assign IP addresses?

DHCP (Dynamic Host Configuration Protocol) is a network protocol that automatically assigns IP addresses and other configuration settings to devices on a network. It operates using a client-server model, where clients request IP addresses from a DHCP server, which allocates them from a pool of available addresses.

- What type of cable should be used when connecting multiple users to an 8-port switch?

Cat6